CSE 240A Homework 2
Pipelines and Memory
Ryan Wang ID:A012345
Due June 1, 2021

Instructions

e Submit typed answers to the following questions as a PDF via Gradescope.
e Due: June 1, 2021 at 11:59 PM.

e Homework is to be done individually.

e There are 2 questions for a total of 75 points (7.5% of your total grade).

e It would be great if you can finish the HW by typing rather than handwriting.

1 1: Pipelining (45 pts)

1. Fill in the pipeline diagram below, assuming it is running on a MIPS 5-stage pipeline with stalls done

 

 

 

 

 

 

 

 

 

 

 

 

 

 

 

 

 

 

 

 

 

 

in decode and forwarding whenever needed. Please clearly show pipeline stalls and forwarding with
arrows.
Note: You may not need to fill in all the cycles in the diagram below.

Instruction 1/2/;3 )4 )5 6 | 7 8 9 10 | 11 | 12 | 13 | 14 | 15

lw R2, 0 (R5) F/D/|X|M| w

sub R10, R2, 10 F|D/D x M)W

add R3, R2, R2 F|F /D X | M Ww

sw R3, 10 (R10) F DX M W

add R2, R2, R5 F/D/|xX mM |W

add R10, R2, R4 F | D x M/W

2. A modified 7-stage pipelined processor has been released with the fetch stage being split into 2 stages:

 

 

  

F1 and F2 and the memory stage is also split into 2 stages: Ml and M2. The memory value is not
available until the end of the M2 stage.
Note: You may not need to fill in all the cycles in the diagram below.

 

3. If the 5-stage processor has a cycle time of 1.75 nanoseconds and the 7-stage processor has a cycle
time of 1.21 nanoseconds, which processor design is faster at executing the code above? State any
assumptions made.

11 * 1.75 = 19.25ns > 14* 1.21 = 16.94ns

So 7-stage processor is faster.